PRODUCT REQUIREMENTS DOCUMENT

Hackathon Project — Smart Infrastructure → Road Safety

Multimodal Driver Fatigue Detection

Using Driving Simulation and Webcam Monitoring

| Category Smart Infrastructure / Road Safety | Format 24–48 Hour Hackathon |
| --- | --- |
| Document Version v1.0 — Initial Release | Date April 2026 |

# Table of Contents

# 1. Executive Summary

Driver fatigue is a leading cause of road accidents worldwide. This project delivers a real-time, multimodal driver fatigue detection system implementable within a 24–48 hour hackathon window. It combines two simultaneous data streams — a web-based driving simulator capturing behavioural telemetry, and a webcam acting as a dashcam capturing physiological signals — and fuses them through a machine learning pipeline to compute a continuous fatigue probability score P(fatigue).

When the score crosses configurable thresholds, the system activates escalating alert mechanisms: controller haptic feedback and mobile phone vibration. This mirrors the architecture used in commercial Advanced Driver-Assistance Systems (ADAS) and smart highway infrastructure.

| Attribute | Value |
| --- | --- |
| Detection Modalities | Driving telemetry + webcam-based physiological signals |
| ML Output | Continuous fatigue probability P(fatigue) ∈ [0, 1] |
| Alert Channels | Game controller haptic + mobile phone vibration |
| Demo Target | Live real-time demonstration in a laptop-based setup |
| Implementation Window | 24–48 hours (hackathon) |

# 2. Problem Statement

## 2.1 The Fatigue Crisis on Roads

Drowsy driving is responsible for an estimated 1 in 5 fatal road collisions globally. Unlike alcohol impairment, fatigue has no breathalyser equivalent — it must be inferred from indirect signals. Current in-vehicle systems either rely on a single modality (eye tracking alone, or lane-keeping alone), which suffer from high false positive rates, or require expensive proprietary hardware.

## 2.2 Why Multimodal Detection Matters

A driver may keep their eyes open while microsleeping (defeating eye-only detection). Equally, a road with many curves may cause natural steering deviation unrelated to fatigue (defeating lane-only detection). Fusing physiological signals with driving behaviour provides a more robust and accurate fatigue estimate, leveraging the complementary nature of both signal types.

## 2.3 Solution Fit

This hackathon project demonstrates the feasibility of a software-only, commodity-hardware fatigue detection system using:

- A standard laptop webcam (as dashcam surrogate)

- A web browser-based driving simulation (as vehicle telemetry surrogate)

- An ML fusion layer running in Python on the local machine or a lightweight server

- Readily available alert channels: USB/Bluetooth game controller and smartphone

# 3. System Architecture

## 3.1 High-Level Architecture Overview

The system is composed of five layers, each independently implementable within the hackathon timeline:

| Layer | Name | Components |
| --- | --- | --- |
| L1 | Input Layer | Webcam feed (30 fps), Driving simulator telemetry (10 Hz JSON stream) |
| L2 | Preprocessing | OpenCV frame decode, MediaPipe FaceMesh, telemetry normalisation |
| L3 | Feature Extraction | EAR, MAR, blink rate, head pose, lane drift, steering instability |
| L4 | ML Inference | Temporal feature fusion → sklearn / PyTorch model → P(fatigue) |
| L5 | Alert Engine | Threshold detection → controller haptic API → phone push notification |

## 3.2 Component Interaction

The Browser (simulation + webcam capture) communicates with the Python backend via a WebSocket connection. The backend runs the ML pipeline and emits fatigue scores at approximately 2 Hz. The alert engine subscribes to these scores and triggers hardware feedback asynchronously.

| KEY DESIGN PRINCIPLE | Both input modalities are captured, processed, and fused in parallel — neither is a fallback for the other. If one modality is unavailable, the system degrades gracefully using the available modality alone. |
| --- | --- |

# 4. Data Pipeline

## 4.1 Camera Pipeline (Vision Modality)

The camera pipeline transforms raw webcam frames into structured numerical features in the following stages:

- Frame Capture — OpenCV VideoCapture reads frames at 30 fps. Frames are resized to 640×480 for processing efficiency.

- Face Detection — MediaPipe Face Detection (BlazeFace) detects the driver's face bounding box. If no face is detected, the frame is skipped and a 'no-face' flag is set.

- Facial Landmark Extraction — MediaPipe FaceMesh returns 468 3D landmarks per frame covering eyes, eyebrows, nose, lips, and face contour.

- Eye Region Extraction — Specific landmark indices (Left eye: 33, 160, 158, 133, 153, 144; Right eye: 362, 385, 387, 263, 373, 380) are used to compute the Eye Aspect Ratio (EAR).

- Mouth Region Extraction — Lip landmarks are used to compute the Mouth Aspect Ratio (MAR) for yawn detection.

- Head Pose Estimation — solvePnP (OpenCV) with a reference 3D face model estimates pitch, yaw, and roll angles from 6 facial landmarks.

## 4.2 Simulation Pipeline (Driving Telemetry Modality)

The JavaScript simulator emits a structured JSON event every 100 ms over a WebSocket connection:

| Field | Type | Description |
| --- | --- | --- |
| lane_offset | float | Signed lateral distance from lane centre (metres) |
| steering_angle | float | Current steering input angle (degrees, −30 to +30) |
| steering_correction_hz | float | Frequency of steering micro-corrections per second |
| reaction_delay_ms | int | Time from lane event trigger to steering response (ms) |
| speed_kmh | float | Current simulated vehicle speed |
| timestamp_ms | int | Epoch timestamp of the telemetry event |

## 4.3 Temporal Aggregation — Sliding Window

Raw per-frame signals are too noisy for direct ML input. Both modalities are aggregated over a 30-second sliding window with a 1-second step. For each window, the following statistics are computed:

- Mean, standard deviation, and trend (linear slope) of each numerical signal

- Blink event count and PERCLOS (percentage of time eyes are >80% closed)

- Lane drift event count (crossings of ±0.3 m threshold)

- Steering reversal count (oscillation proxy for fatigue-related over-correction)

# 5. Feature Extraction

## 5.1 Vision Features

| Feature | Abbrev. | Fatigue Significance |
| --- | --- | --- |
| Eye Aspect Ratio | EAR | Drops below 0.2 during heavy blinks or microsleep. PERCLOS derived from EAR threshold crossings. |
| Blink Frequency | BF | Increases up to ~30 bpm in early fatigue; drops sharply in microsleep episodes. |
| Eye Closure Duration | ECD | Closures >500 ms strongly correlated with microsleep. Key binary event trigger. |
| Mouth Aspect Ratio | MAR | Spikes above 0.6 during yawning — a strong physiological fatigue marker. |
| Yawn Frequency | YF | >3 yawns per 5 minutes indicates significant fatigue onset. |
| Head Pitch Angle | HP | Forward head drop (>15°) indicates nodding off. Lateral tilt can indicate loss of attention. |
| Gaze Direction | GD | Extended off-road gaze from FaceMesh iris landmarks — a distraction/fatigue proxy. |

## 5.2 Driving Behaviour Features

| Feature | Fatigue Significance |
| --- | --- |
| Lane Drift Variance | High variance in lane offset indicates loss of motor control from fatigue |
| Lane Offset Mean | Systematic bias indicates drift to one side — correlated with late-stage fatigue |
| Steering Instability Index | Std deviation of steering angle divided by mean absolute value |
| Steering Correction Frequency | Frequent micro-corrections signal the driver is actively fighting drowsiness |
| Reaction Delay (ms) | Increased response time to lane trigger events is a proven fatigue marker |
| Steering Reversal Rate | Count of sign changes in steering angle per second |

## 5.3 Context Features

- Session Duration (minutes) — Fatigue risk scales with driving time

- Time of Day — Circadian rhythm dips at 2–4 AM and 1–3 PM amplify fatigue probability

- Simulated Road Complexity — Straight road baselines vs curve-heavy sections

# 6. Multimodal Feature Fusion

## 6.1 Feature Vector Construction

At each inference step (every 1 second), the system assembles a single flat feature vector by concatenating the outputs of both pipelines:

| FEATURE VECTOR | F = [EAR_mean, EAR_std, EAR_trend, BF, ECD_max, MAR_max, YF, HP_mean, HP_std, GD_ratio, lane_drift_var, lane_offset_mean, steering_instability, correction_freq, reaction_delay_mean, steering_reversals, session_duration, time_of_day_sin, time_of_day_cos] → dim = 19 |
| --- | --- |

Time-of-day is encoded as a sine/cosine pair to preserve its cyclic nature (e.g., 23:00 is close to 01:00).

## 6.2 Why Multimodal Fusion Improves Accuracy

Each modality has distinct failure modes that the other compensates for:

| Scenario | Vision-Only Result | Multimodal Result |
| --- | --- | --- |
| Driver opens eyes but is microsleeping | ❌ Missed — eyes appear open | ✅ Detected via lane drift spike |
| Glare / low light obscures face | ❌ No signal available | ✅ Driving telemetry maintains detection |
| Curved road causes normal steering oscillation | ✅ Correctly flagged as alert | ✅ Low EAR/MAR prevents false positive |
| Driver is awake but distracted (phone) | ❌ Eyes open — no alert | ✅ Gaze deviation + mild lane drift triggers alert |

# 7. Machine Learning Pipeline

## 7.1 Model Architecture Options

Given the hackathon timeline, the recommended primary model is a Gradient Boosting Classifier (LightGBM or XGBoost) trained on the 19-dimensional feature vector. The table below compares candidate architectures:

| Model | Hackathon Feasibility | Expected AUC | Notes |
| --- | --- | --- | --- |
| Random Forest | ✅ Very High | 0.82–0.87 | Easy to train, good baseline, interpretable feature importance |
| LightGBM / XGBoost | ✅ High | 0.87–0.92 | RECOMMENDED — fast training, excellent tabular performance, SHAP support |
| MLP (PyTorch) | ✅ Medium | 0.85–0.90 | Viable if pretrained on public dataset; requires more tuning |
| LSTM (PyTorch) | ⚠️ Low | 0.88–0.93 | Best for temporal patterns but requires sequence batching — post-hackathon |
| Transformer (ViT) | ❌ Very Low | 0.90–0.95 | Best-in-class but prohibitive to train in 48 hours without GPU + data |

## 7.2 Training Strategy

### 7.2.1 Data Sources

Since collecting labelled fatigue data in 48 hours is infeasible, the following public datasets should be used for pre-training:

- DROZY Dataset — Physiological drowsiness signals with ground truth labels

- UTA-RLDD — Real-life drowsiness detection with facial video

- NTHU Driver Drowsiness Detection Dataset — Multi-condition drowsiness video

- Synthetic augmentation — use OpenFace to generate EAR/MAR sequences with known labels

### 7.2.2 Training Pipeline

- Preprocess all dataset frames through the same FaceMesh pipeline to extract feature vectors

- Label encode: alert=0, mildly drowsy=1, moderately drowsy=2, severely drowsy=3

- Train binary classifier (alert vs drowsy) and regression model (0.0–1.0 probability)

- Calibrate probability outputs using Platt scaling to ensure P(fatigue) is well-calibrated

- Evaluate on held-out test split with cross-subject validation

## 7.3 Model Output

The model outputs a single scalar: P(fatigue) ∈ [0, 1]. This is a calibrated probability — not a raw logit — enabling direct threshold comparison in the alert engine.

# 8. Fatigue Decision Engine

## 8.1 Threshold Definitions

The fatigue probability score is mapped to four operational states with escalating alert responses:

| Level | P(fatigue) | User-Facing Response | Alert Channel |
| --- | --- | --- | --- |
| Normal | < 0.30 | No action — continue monitoring | Haptic + Visual |
| Mild Fatigue | 0.30 – 0.55 | Soft audio chime + mild controller rumble | Haptic + Visual |
| High Fatigue | 0.55 – 0.75 | Sustained vibration + on-screen warning banner | Haptic + Visual |
| Critical Fatigue | > 0.75 | Full phone + controller haptic loop + voice alert + simulation pause | Haptic + Visual |

## 8.2 Hysteresis Logic

To prevent alert flickering (rapid on/off transitions at a threshold boundary), a hysteresis mechanism is applied. Once a threshold is crossed upward, the system requires the score to drop 0.08 below the boundary before deactivating the corresponding alert level. This mirrors the Schmitt trigger design used in automotive ECU alarm systems.

## 8.3 Temporal Smoothing

The raw per-second ML output is smoothed with an exponential moving average (EMA) with alpha=0.3 before threshold comparison. This suppresses single-frame noise while maintaining sub-3-second response latency to genuine fatigue onset.

# 9. Alert System

## 9.1 Alert Channel 1 — Controller Haptic Feedback

When the fatigue score exceeds the Mild Fatigue threshold, the game controller's rumble motors are activated via the Gamepad Vibration API (Web Vibration API for browser, or pygame for Python):

- Mild Fatigue: 200 ms pulse every 3 seconds at 40% intensity

- High Fatigue: 500 ms pulse every 1.5 seconds at 70% intensity

- Critical Fatigue: Continuous 80% intensity dual-motor rumble

## 9.2 Alert Channel 2 — Mobile Phone Vibration

Phone vibration is triggered via one of the following mechanisms (choose based on hackathon setup):

- OPTION A: PWA Vibration API — the simulation runs as a Progressive Web App on a phone, using navigator.vibrate()

- OPTION B: WebSocket push to phone — A lightweight React Native app subscribes to fatigue score updates and calls Vibration.vibrate()

- OPTION C: Bluetooth LE — FastAPI backend sends a BLE characteristic update to a paired phone running a BLE listener app

## 9.3 On-Screen Visual Alerts

The React dashboard overlays progressive warning banners on the simulation:

- Mild Fatigue: Yellow banner — 'Fatigue detected. Take a break soon.'

- High Fatigue: Orange pulsing banner — 'WARNING: High fatigue. Pull over.'

- Critical Fatigue: Red full-screen overlay with countdown — simulation auto-pauses after 5 seconds

| ALERT PERSISTENCE | All alerts remain active until P(fatigue) drops below the respective threshold minus hysteresis margin. They cannot be dismissed by the user during the demo — this is intentional for safety-critical system simulation. |
| --- | --- |

# 10. Technical Pipeline Flowchart

## 10.1 Step-by-Step Pipeline

| STEP 1 | Input Capture | Webcam captures face at 30 fps via navigator.mediaDevices.getUserMedia(). Simulator publishes telemetry JSON at 10 Hz over WebSocket (ws://localhost:8000/ws/telemetry). |
| --- | --- | --- |

| STEP 2 | Frame Routing | Browser sends webcam frames (JPEG, quality=0.7) and telemetry events to FastAPI backend via WebSocket. |
| --- | --- | --- |

| STEP 3 | Vision Preprocessing | FastAPI decodes JPEG → numpy array. OpenCV resizes to 640×480. MediaPipe FaceMesh extracts 468 landmarks. |
| --- | --- | --- |

| STEP 4 | Vision Feature Computation | Python computes EAR, MAR, head pose (solvePnP), gaze direction ratio from landmark coordinates. Appends to rolling 30-second buffer. |
| --- | --- | --- |

| STEP 5 | Telemetry Preprocessing | FastAPI parses JSON telemetry. Computes rolling statistics (mean, std, trend) for each telemetry field over the last 30 seconds. |
| --- | --- | --- |

| STEP 6 | Feature Vector Assembly | At each 1-second tick, both 30-second rolling statistics are concatenated into the 19-dim feature vector F. |
| --- | --- | --- |

| STEP 7 | ML Inference | F is passed to the pre-trained LightGBM model. predict_proba() returns P(fatigue). EMA smoothing is applied. |
| --- | --- | --- |

| STEP 8 | Threshold Evaluation | Smoothed P(fatigue) is compared to thresholds with hysteresis. Fatigue state is updated (Normal / Mild / High / Critical). |
| --- | --- | --- |

| STEP 9 | Alert Dispatch | Alert engine sends WebSocket message to browser (visual alert), triggers controller rumble via Gamepad API, and sends push to phone (via chosen channel). |
| --- | --- | --- |

| STEP 10 | Dashboard Update | React dashboard receives P(fatigue) score and fatigue state. Updates gauge, EAR/MAR time-series chart, and simulation overlay. |
| --- | --- | --- |

# 11. Recommended Technology Stack

| Domain | Technology | Justification |
| --- | --- | --- |
| Computer Vision | OpenCV 4.x | Industry-standard frame processing; seamless MediaPipe integration |
| Facial Landmarks | MediaPipe FaceMesh | 468-point real-time landmarks at 30+ fps on CPU — no GPU required |
| Head Pose | OpenCV solvePnP | Accurate 6-DOF pose from 6 facial landmarks using PnP algorithm |
| ML Framework | LightGBM + scikit-learn | Fast tabular training; SHAP for explainability; sklearn pipeline support |
| Deep Learning | PyTorch 2.x | Optional LSTM upgrade path; torchscript for efficient inference |
| Backend API | FastAPI + uvicorn | Async WebSocket support; auto OpenAPI docs; Python ecosystem compatibility |
| Driving Simulation | Three.js / WebGL | Browser-native 3D; no install required; easy telemetry event emission |
| Frontend Dashboard | React + Recharts | Real-time score gauge; EAR/MAR chart; simulation overlay components |
| Controller API | Gamepad API (Web) | Native browser API for controller rumble — no driver install needed |
| Mobile Alerts | React Native / PWA | Cross-platform phone vibration; WebSocket subscription to alert events |
| Data Processing | NumPy + Pandas | Sliding window feature aggregation; fast vectorised statistics |
| Model Serving | FastAPI inference endpoint | predict() called inline in WebSocket handler — sub-10 ms latency |

# 12. Demo Architecture & Plan

## 12.1 Physical Setup

The demo requires a single laptop with the following connections:

- Built-in or USB webcam facing the participant — acting as dashcam

- USB game controller (Xbox / PlayStation) connected for haptic feedback

- Smartphone connected to the same WiFi network for push vibration alerts

- Browser tab open with the driving simulation + React dashboard

## 12.2 Demo Flow

| 0:00 | Setup | Participant sits in front of laptop. Webcam calibrates. Simulation loads. |
| --- | --- | --- |

| 0:30 | Baseline | Participant drives normally. P(fatigue) stays in Normal zone (green). |
| --- | --- | --- |

| 2:00 | Fatigue Induction | Participant is asked to simulate fatigue: slow blinks, slight head drop, reduced steering precision. |
| --- | --- | --- |

| 2:30 | Mild Alert | P(fatigue) crosses 0.30. Controller pulses. Dashboard turns amber. Phone vibrates once. |
| --- | --- | --- |

| 3:30 | High Alert | P(fatigue) crosses 0.55. Sustained rumble. Orange warning banner appears on simulation. |
| --- | --- | --- |

| 4:30 | Critical Alert | P(fatigue) crosses 0.75. Full red overlay. Simulation pauses. Phone vibrates continuously. |
| --- | --- | --- |

| 5:00 | Recovery | Participant returns to alert state. Scores drop. Alerts deactivate progressively. |
| --- | --- | --- |

# 13. Evaluation Metrics

## 13.1 ML Model Metrics

| Metric | Definition & Target |
| --- | --- |
| AUC-ROC | Primary metric — measures model's ability to discriminate alert vs fatigued across all thresholds. Target: >0.87 |
| AUPRC | Precision-Recall AUC — important given class imbalance (fatigue events are rare). Target: >0.80 |
| F1 Score (High+Critical) | Harmonic mean of precision and recall for dangerous fatigue classes. Target: >0.82 |
| False Positive Rate | Rate of unnecessary alerts — directly affects driver trust. Target: <15% at Mild threshold |
| False Negative Rate | Missed critical fatigue events — safety-critical. Target: <5% at Critical threshold |
| Detection Latency | Time from fatigue onset to alert trigger. Target: <4 seconds end-to-end |
| Inference Latency | Time for single ML prediction cycle. Target: <100 ms per inference step |

## 13.2 System Performance Metrics

- WebSocket round-trip latency: < 50 ms (localhost)

- FaceMesh frame rate: > 25 fps on mid-range laptop

- End-to-end alert trigger latency: < 4 seconds from fatigue onset

- Controller haptic response latency: < 100 ms from alert state change

- System CPU usage: < 70% on dual-core CPU (leaving headroom for simulation)

# 14. Risks and Challenges

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Low-light / glare on face | HIGH | Apply histogram equalisation + CLAHE. Set minimum face detection confidence threshold. Fallback to driving-only mode if face confidence < 0.6. |
| Real-time inference latency spike | HIGH | Profile MediaPipe bottleneck. Skip every 3rd frame for FaceMesh. Run ML inference on background thread. Buffer telemetry events. |
| Simulation-webcam synchronisation drift | MEDIUM | Timestamp all events with server-side epoch. Align using nearest-neighbour matching within 500 ms window. |
| Controller API not supported (browser) | MEDIUM | Fall back to keyboard simulation + screen flash alerts. Test Gamepad API compatibility on Chrome before demo. |
| Dataset domain gap (lab vs demo) | MEDIUM | Fine-tune model on 5-minute self-recorded demo data. Use calibration phase at session start to normalise EAR baseline. |
| Phone WiFi connectivity drop | LOW | Implement alert queue with 10-second retry. Visual dashboard alerts are always available as fallback. |
| Glasses / sunglasses blocking iris | LOW | Use outer eye landmarks only (EAR from eyelid, not iris). MediaPipe FaceMesh handles glasses in most conditions. |

# 15. Future Scope

## 15.1 Real Vehicle Telemetry Integration

Replace the web simulator with OBD-II dongle data from a real vehicle. CAN bus provides steering angle, speed, lane departure warning signals, and accelerometer data at up to 100 Hz. This makes the system deployable as a true ADAS component.

## 15.2 Edge AI Deployment

The ML model can be exported to ONNX and quantised to INT8 for deployment on Raspberry Pi 4 or NVIDIA Jetson Nano, embedded in the vehicle dashboard. MediaPipe Lite and TFLite variants run at 15+ fps on ARM devices.

## 15.3 Smart Highway Infrastructure Integration

Aggregated anonymised fatigue events from a vehicle fleet can be streamed to a highway management platform. Heatmaps of high-fatigue road segments (e.g., long straight motorways at 2–4 AM) can inform dynamic speed limit adjustments and roadside rest stop signage.

## 15.4 LSTM / Transformer Upgrade

With sufficient labelled data (>10,000 sessions), replacing the tabular model with an LSTM or Temporal Convolutional Network trained on raw 30-second feature sequences would capture fatigue progression patterns more accurately, particularly gradual microsleep onset.

## 15.5 Multi-Occupant Monitoring

Extend the webcam pipeline to track multiple faces (e.g., a bus or truck cab with multiple occupants) using MediaPipe Multi-Face Detection. Independent fatigue scores per detected face, with cabin-level aggregate alert.

## 15.6 Physiological Sensor Fusion

Integrate wearable biometrics — heart rate variability (HRV), galvanic skin response (GSR), and EEG alpha-wave power — as additional modalities. These are the gold standard for fatigue research and would significantly improve model accuracy beyond the webcam-only baseline.

# 16. 48-Hour Hackathon Implementation Timeline

| Milestone | Deliverable |
| --- | --- |
| Hour 0–2 | Environment setup — Python venv, FastAPI, MediaPipe, LightGBM, Three.js boilerplate |
| Hour 2–6 | Build camera pipeline — FaceMesh integration, EAR/MAR computation, head pose |
| Hour 6–10 | Build driving simulator — Three.js scene, lane markings, keyboard/controller input, telemetry WebSocket |
| Hour 10–14 | Build feature extraction + sliding window aggregation for both modalities |
| Hour 14–18 | Train ML model on public dataset features — LightGBM baseline + calibration |
| Hour 18–22 | Build alert engine — threshold logic, controller haptic, phone vibration |
| Hour 22–28 | Build React dashboard — fatigue gauge, EAR/MAR charts, simulation overlay, alert banners |
| Hour 28–34 | End-to-end integration testing — WebSocket sync, latency profiling, edge case handling |
| Hour 34–40 | Demo rehearsal — fatigue induction scenario, alert escalation verification |
| Hour 40–48 | Presentation preparation — slides, narrative, live demo polish |